| Republic of Serbia | (Coat of Arms) Belgrade www: Beograd.rs | 43, 27 marta Street |
| --- | --- | --- |
| City of Belgrade |  | 11000 Belgrade |
| City Administration of the City of Belgrade |  | Phone (011) 2754, Fax 2754-636 |
| Secretariat for Transport |  | e-mail: info.saobracaj@beograd.gov.rs |
| Sector for Transport Planning and Urban Mobility |  |  |
| Department for Transport Planning |  |  |
| IV-08 No. 344.16-1047/2024 |  |  |
| February 14, 2025 |  |  |

Urban Planning Institute of Belgrade

56, Bulevar despota Stefana Street

11000 Belgrade

Your ref: 350-1453/24

Further to your request for issuing contest conditions for the purpose of invitation to the Open, International, Poll-Based, One-Stage, Anonymous Contest for the Architectural and Urban Solution of a Part of Savski Square in the Hinterland of the Building of the Old Railway Station in Belgrade, the Secretariat for Transport is submitting the following conditions:

- The alignment line of the streets adjacent to the area covered by the Contest to be taken over from the Spatial Plan for the Special Purpose Area of Development of a Part of the Riverside of the City of Belgrade – the Riverside Area of the Sava River for the Belgrade Waterfront Project (Official Gazette of the City of Belgrade, Nos. 7/15, 48/22) hereinafter: the SPSPA.

It should be taken into account that for implementation of the Contest Solution for the squares in question it is necessary to realize the streets (adjacent to the area in question) in accordance with the valid plan.

The plot in question is located within the scope of the SPSPA and the General Regulation Plan of Rail Systems in Belgrade with the Elements of Detailed Elaboration for the Phase I of the First Line of Metro System (Official Gazette of the City of Belgrade, No. 102/21) (hereinafter: the GRP of Rail Systems).

In the SPSPA, in the part relating to implementation of the plan, it is defined that the GRP of Rail Systems is effective and is implemented within the scope of the plan.

The building plot – cultural institution (GP J9-19) is defined according to the GRP of Rail Systems and it is stated that it is not permitted to change the defined building plots. A part of this building plot is within the scope of the contest in question.

- According to the SPSPA, vehicle and pedestrian access to the building plot GP11a is possible from the street SAO6 and Karadjordjeva Street, and according to the GRP of Rail Systems vehicle access is possible from the street SAO6.

Vehicle accesses to be designed at a distance of at least 15.00m from the intersection, measured from the alignment line of the cross street according to the principle entry/exit (without crossing the divisional island, only right turns), in order to reduce conflict points and adverse effects on the flow within the observed street network.

- As within the contest site it is necessary to provide access of heavy goods vehicle (HGV) for delivery of exhibits (and if there is need of access of tourist buses), for the plot on which the Museum is located (across the square) dimensioning of the accesses is necessary in relation to:

- design vehicle, i.e. relative to a vehicle with poorer/more demanding maneuvering capabilities, and

- depending on the width of the street from which it is accessed,

so that the passability conditions for the design vehicle are satisfied, in such a way that the vehicle can enter/exit the plot driving forwards, without any additional maneuvering on the street.

- It is possible to design vehicle accesses at the level of carriageway. On the part of the “fan” vehicle access which is crossing the sidewalk, flush curbs to be provided in the width of the sidewalk, in order for pedestrian movement within the sidewalk of the street SAO6 and Karadjordjeva Street to remain continuous.

- The area on the plot intended for movement of vehicles along the plot and vehicle maneuvering, must be constructed with a base adapted to movement of vehicles and dimensioned according to the expected traffic load (asphalt/concrete).

It is recommended to clearly separate (physically and visually) the internal street (area for movement of vehicles within the square) from the area for movement of vehicles, for example, by a different color and materialization, bollards, high curbs, etc.).

- If it is planned to install the plot access control system, it is compulsory to reserve the front part of the pertaining plot so that a vehicle waiting to access does not obstruct traffic flow on the surrounding street network.

The internal street in the part where access (entrance/exit) control is done to be designed with maximum longitudinal grade of 2.5%.

- All areas within the contest area intended for vehicle movement must satisfy passability conditions (traffic lane widths, curve radii, longitudinal grades, free heights, etc.) for the adopted design vehicle (delivery/goods and/or utility vehicle, fire truck, bus), depending on the planned vehicle movement pattern.

Internal streets for movement of passenger cars to be planned with minimum traffic lane width of 2.75m, and for goods vehicles 3.50m.

- Delivery scheme to be elaborated and delivery to be planned so as not to obstruct the surrounding street network.

- A plateau for temporary stopping of vehicles is to be planned if, in accordance with the technological processes, it is necessary on the planned plot to provide stopping of heavy goods vehicles (HGVs) for delivery of exhibits (or tourist buses).

- As the depot for storing the exhibits is located in the Museum building, if necessary, a corridor/communication from the loading/unloading area to the depot to be provided. The said corridor must be made with the base adapted to movement of the design vehicle (fork-lift truck), which is used for loading/unlading and transporting the exhibits. Dimensioning of the said corridor to be made according to the expected traffic load.

As regards situation and levelling, the said corridor to be integrated relative to the planned pedestrian flows on the plot concerned.

- Based on demand, there is a need for access of passenger cars and taxis within the contest site. According to the valid planning documents (GRP of Rail Systems and SPSPA), vehicle access from Savska Street is not permitted. As preparation of amendments to the SPSPA is ongoing, it is proposed by this plan to examine and develop in detail the possibility of opening vehicle access from Savska Street. After considering the situation in detail the Secretariat for Transport did not agree with the proposal, i.e. with opening a vehicle access for the “drop-off” point from Savska Street at the proposed site between the old Railway Station building (Historical Museum of Serbia) and the old Post Office building (Children’s Museum of Belgrade) which would enable the users to have a short term access to the area concerned.

- The part of the plateau intended for stopping of taxis (and tourist buses, if planned) to be designed so that during exiting/entering the vehicle passengers are in the area intended for pedestrian movement.

- Since plateaus for movement and stopping of motor vehicles are planned within the plot concerned it is necessary to design pedestrian crossings at the place where pedestrian communications and the carriageway are intersecting.

- Pedestrian areas to be designed in accordance with the Rulebook on Technical Standards of Planning, Designing and Construction of Buildings, which Ensure Unhindered Movement to Persons with Disabilities, Children and Elderly (Official Gazette of RS, No. 22/2015).

- When choosing the material for paving the square surface care should be taken that the material does not radiate excessively and that it reduces the risk of slipping.

- If any grids, manholes or covers are installed within the areas intended for pedestrian movement, they should be installed in such a way to ensure safe movement of pedestrians, particularly persons with disabilities.

- If it is planned to plant tree seedlings they should be designed in such a way not to interfere with unhindered movement of pedestrians, particularly persons with disabilities.

- In the width of pedestrian/cycle crossings flush curbs (h =0cm) to be planned.

- In the area covered by the contest bicycle paths should be planned and continuity in guidance of bicycle traffic should be enabled relative to the contact area (existing and planned paths). Bicycle paths to be planned with minimum width of 2.5m and guided independently of the carriageway (min 0.75m from the outer carriageway edge). Bicycle path clearance in the height of 2.50m along the entire bicycle path surface to be saved.

- Installation of P style racks for bicycle parking to be planned in the vicinity of the cultural facility.

- Within Savski Square a space to be planned for bicycle rental stations defined by the Plan of Locations for Installing Bicycle Rental Stations in the Territory of the City of Belgrade (location No. 66). Micro location of the bicycle rental station to be elaborated in detail through this contest.

Space for two elements to be reserved with minimum dimensions of 5x4m with vertical clearance of 2.75m (such space may be on one or two locations, depending on the spatial possibilities). Accurate dimensions will be subsequently determined when exact appearance of the station is established.

- By inspecting valid planning documents, General Regulation Plan of Rail Systems in Belgrade with the Elements of Detailed Elaboration for the Phase I of the First Line of Metro System (Official Gazette of the City of Belgrade, No. 102/21), it was found that within the location concerned there is Savski Square metro station and that the route of the first and second metro line (with safety zone of stretching route) is running there.

The requirements in connection with construction in the metro route zone to be procured from PUC Belgrade Metro and Train.

- The urban and architectural solution of the part of Savski Square in question (in the part relating to metro stations) to be made in accordance with the design for construction of the phase I of line 1of the Belgrade Metro.

Prepared by: Jelena Crnogorac, MSc (Traffic)

|  | Acting Deputy Head of City Administration of the City of Belgrade Bojan Bovan, LLB |
| --- | --- |